Sec. 30-152. Owner's responsibility for sewer lines.

(a)   All house sewers from the property line to the building are the property and responsibility of the property owner.

(b)   All stoppage in house sewers from sewer gravity main to property line, except those caused by broken pipe or roots from city-owned trees, are the responsibility of the property owner.

(c)   No stoppage complaint will be accepted for investigation by the city manager unless all pipes, drains and sewers between the sewer gravity main and the building have been examined by a licensed plumber who has certified to the city manager that stoppage is caused by broken pipe or roots from city-owned trees between the property line and the sewer gravity main.

(d)   Where stoppage between the sewer gravity main and the property line is caused by broken pipe or roots from city-owned trees, the city manager will clean and repair that section between the property line and the sewer gravity main.

(e)   Additional standards are as follows:

(1)   Size.  The minimum allowable size for any private line running into the city wastewater gravity main shall be not less than 6 inches in diameter, and any service line with 3 or more 4-inch connections on it shall be not less than 6 inches in diameter. The lateral from the property line to the city main shall be a minimum of 6 inches in diameter. All gravity main shall be a minimum of 8 inches in diameter.

(2)   Inspection.  All material and workmanship shall be approved by the city manager before acceptance by the city.

(3)   Discharge into floodwaters.  New or replacement sanitary sewage collection systems shall be designed to eliminate discharges from the systems into floodwaters.

(Code 1994, § 66-82; Ord. No. 94-7187, § 3, 5-4-1994)